Firmware, Web UI, and Default Configuration Update SOP

Step 1. Preparation

- Connect the PC to the switch via the console port and management port.

- On the PC desktop, create a new folder and unzip the firmware package into this folder.
The extracted files include:

  - Firmware: FW_SGS-6310-48P6XR_2.2.0F_Build_142241.bin

  - Web UI: SGS-6310-Series_MQTT_web.wrp

  - Default configuration: SGS-6310-48P6XR_default-config

- Launch the Tftpd64 utility and configure:

  - Current Directory: [path to the unzipped folder]

  - Server Interface: [management port IP address]

Step 2. Access the Command Line Interface

- Execute Tera Term or PuTTY to connect to the switch console.

- Set the PC’s IP address to the 192.168.1.x network segment.

（1）Update Firmware

Step 1. After powering on the switch, wait until the boot message displays ‘SDRAM Fast Test……..PASS’, then press Ctrl+P to enter Management Mode.

Step 2. Enter Management Mode and enter the following commands in sequence.

monitor#ip add 192.168.1.22 255.255.255.0

monitor#copy tftp flash

Source file name[]?FW_SGS-6310-48P6XR_2.2.0F_Build_142241.bin

Remote-server ip address[]?192.168.1.20

Destination file name[]?switch.bin

Step 3. Complete the firmware update process.

（2）Update Web UI

Step 1. After powering on the switch, wait until the boot message displays ‘SDRAM Fast Test……..PASS’, then press Ctrl+P to enter Management Mode.

Step 2. Enter Management Mode and enter the following commands in sequence.

monitor#copy tftp flash

Source file name[]?SGS-6310-Series_MQTT_web.wrp

Remote-server ip address[]?192.168.1.20

Destination file name[web.wrp]?web.wrp

monitor#reboot

Do you want to reboot the Switch(y/n)?y

Step 3. Complete the Web UI update process.

（3）Update Default Config

Step 1. After powering on the switch, wait until the boot message displays ‘SDRAM Fast Test……..PASS’, then press Ctrl+P to enter Management Mode.

Step 2. Enter Management Mode and enter the following commands in sequence.

monitor#ip add 192.168.1.22 255.255.255.0

monitor#show filesys-backdoor

monitor#copy tftp flash

Source file name[]?SGS-6310-48P6XR_default-config

Remote-server ip address[]?192.168.1.20

Destination file name[default-config]?default-config

Step 3. Complete the Default Config update process.